Digital Design — Summer Project

FPGA Implementation of the IEEE 802.15.4 Chirp Spread Spectrum (CSS) Physical Layer Transmitter

## 1. Introduction and Motivation

IEEE 802.15.4 is the physical (PHY) and medium access control (MAC) standard behind low-rate, low-power wireless personal area networks (LR-WPANs), including ZigBee. Among the alternative physical layers added to the standard, the Chirp Spread Spectrum (CSS) PHY, operating at 2450 MHz, is particularly interesting from a digital design point of view: it combines block coding, interleaving, differential QPSK modulation, and a chirp-based modulation scheme (CSK) that must be generated numerically and implemented efficiently in hardware.

This project asks you to design, implement, and verify a synthesizable Verilog implementation of the CSS PHY transmitter chain described in the attached reference document ("Zibee Reference Document"), following the same design flow used in industry: floating-point reference model → fixed-point model → RTL (Verilog) design → simulation/verification → FPGA synthesis and implementation → on-board verification.

You will be given, as a starting point, a MATLAB floating-point reference model and a MATLAB fixed-point (quantized) model of the CSS transmitter. Your task is to translate the fixed-point model into synthesizable Verilog, verify it against the MATLAB reference, and demonstrate it running on an FPGA board.

## 2. Learning Objectives

- Translate a DSP/communications algorithm described in a standard and a MATLAB model into a synthesizable RTL architecture.

- Practice fixed-point (quantized) arithmetic design: choosing word lengths, rounding/truncation, and verifying accuracy against a floating-point golden reference using Mean Square Error (MSE).

- Design and integrate multiple interacting RTL blocks (datapath + control) using a top-down, modular methodology.

- Write and use HDL testbenches for unit-level and integration-level verification.

- Use behavioral simulation, synthesis, implementation, and device programming as part of a complete FPGA design flow.

- Communicate a hardware design through documentation: block diagrams, interface tables, and verification reports.

## 3. Background Summary (see reference document for full detail)

The following is a condensed summary of the technical background you must understand before starting implementation. Full details, tables, and equations are in the attached Zibee Reference Document; you are expected to read it carefully, particularly Chapters 1 and 2.

### 3.1 IEEE 802.15.4 CSS PHY frame (PPDU) structure

The CSS PHY protocol data unit (PPDU) is composed of a Synchronization Header (preamble + Start-of-Frame Delimiter, SFD), a PHY Header (PHR) that encodes the payload length, and the PHY payload (PSDU, up to 127 bytes). Table 2-1 of the reference document gives the exact symbol counts for the 1 Mbps and 250 kbps data rates.

### 3.2 Transmitter processing chain

The floating-point transmitter model (Figure 2-1 of the reference) implements the following sequential processing blocks. Your Verilog design must implement the equivalent fixed-point datapath for each of these blocks:

- Zero padding — pads the PHR + PSDU bit stream so its length is a multiple of N (N = 6 for 1 Mbps, N = 24 for 250 kbps).

- Demux — splits the padded bit stream into even bits (I path) and odd bits (Q path).

- Symbol Mapper — block-encodes each group of n bits into an m-bit Walsh–Hadamard codeword (n=3, m=4 for 1 Mbps; n=6, m=32 for 250 kbps), per the code tables in the IEEE 802.15.4 standard (Tables 26a/26b).

- Interleaver — reorders 16 groups of 4 bits each (250 kbps only; bypassed for 1 Mbps) according to the fixed permutation given in the reference document.

- Form PPDU — merges preamble, SFD, PHR, and the zero-padded/encoded payload into the final I/Q bit sequence.

- QPSK Mapper — maps each 2-bit (I,Q) pair to one of the 4 QPSK constellation points.

- Differential QPSK (DQPSK) Coding — computes Sₙ = Xₙ·Sₙ₋₄, i.e., each QPSK symbol is multiplied by the symbol four positions earlier, with S₀=S₁=S₂=S₃=exp(jπ/4) as initial values.

- CSK Generation and Modulation — the key DSP block: each group of 4 successive DQPSK symbols selects one of 4 possible 152-sample Chirp Sequences (index m = 1..4), built from 4 sinusoidal "subchirps" (38 samples each, fs = 32 MHz) shaped by a raised-cosine window (roll-off α = 0.25) and separated by a data-dependent time gap (Teven/Todd).

### 3.3 Fixed-point conversion and accuracy target

The reference document shows (Section 3.1) that the only real source of "floating-point-ness" in the design is the chirp sequence generator, since every other block is already bit/integer-oriented. The reference analysis sweeps the ADC/quantizer resolution R from 2 to 8 bits and computes the Mean Square Error (MSE) between exact (floating-point) and fixed-point chirp samples:

meanSquareError = Σ|exactValue − fixedValue|² / Σ|exactValue|² over N = 152 samples (one full chirp sequence).

The accepted MSE threshold from the standard (Section 6.5a.5.1 of [8]) is 0.005. The reference design found that any resolution R > 4 bits satisfies this, and selected R = 5 bits as the best trade-off between accuracy, complexity, and power. You will use the same acceptance criterion (or one specified by your instructor) to justify your own choice of internal word lengths.

### 3.4 Reference transmitter architecture

Figure 3-3 of the reference document shows one valid hardware architecture for the transmitter: a payload RAM (127×8 bits) feeding a demux into I/Q paths, each with a Symbol Mapper ROM and an I/Q buffer RAM, a Controller that sequences preamble/SFD/PHR/payload data, a QPSK mux/mapper, a 4-stage delay (4 flip-flops) with multiplier for DQPSK coding, and a CSS Symbols ROM with a final multiplier to produce the modulated output. You may use this architecture as a starting point, or propose your own, as long as interfaces and behavior match the specification below.

## 4. What You Are Given

- The Zibee Reference Document (this PDF) — the required reading covering IEEE 802.15.4, CSS PHY theory, the floating-point algorithm, and one possible hardware architecture.

- A MATLAB floating-point reference model implementing the transmitter chain exactly as in Section 2 of the reference document (bit-accurate golden reference for correctness).

- A MATLAB fixed-point model (using the Fixed-Point Toolbox, `fi` objects) implementing the same chain with quantized word lengths, which is the direct specification for your RTL. This model also includes a test-vector generator you can use to produce input payloads and expected outputs.

- A short IEEE 802.15.4a-2007 excerpt with the official Walsh–Hadamard code tables (26a/26b) referenced in Section 2.1.3.

## 5. Project Requirements / Deliverable Design

You must design and implement, in synthesizable Verilog, a CSS PHY transmitter with the interface below (adapted from Table 3-1 of the reference document). You may add internal signals as needed, but the external interface must match so your design can be tested with the provided testbenches.

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| clk | 1 | IN | System clock |
| reset | 1 | IN | Synchronous system reset |
| start_Tx | 1 | IN | Pulse from MAC to PHY to initiate transmission |
| payloadLength | 8 | IN | Payload length in bytes (≤ 127) |
| payload_din / addr | 8 | IN | Payload data written to the payload RAM before start_Tx |
| done_Tx | 1 | OUT | Pulses high when the whole PPDU has been transmitted |
| Tx_real | 8 | OUT | Real part of the modulated CSS output sample |
| Tx_imag | 8 | OUT | Imaginary part of the modulated CSS output sample |

### 5.1 Mandatory sub-modules

Your design must be modular, with one Verilog module per functional block, mirroring Figure 2-1 / 3-3 of the reference document:

- Zero-padding / payload framer

- Demux (I/Q bit splitter)

- Symbol Mapper (ROM-based Walsh–Hadamard encoder, one instance per I/Q path)

- Interleaver (250 kbps path — may be a simple wired permutation, not necessarily a ROM)

- Preamble/SFD generator and PPDU framer / controller (FSM)

- QPSK Mapper

- DQPSK differential encoder (4-symbol delay + complex multiplier)

- CSK Generator / chirp-sequence ROM and modulator (complex multiplier)

- Top-level transmitter module exposing the interface in Section 5

### 5.2 Design parameters you must choose and justify

- Fixed-point word length and format (integer/fraction bits) for I/Q samples, internal multiplier outputs, and the chirp ROM, with an MSE-based justification as in Section 3.1.3 of this document.

- Whether to support one or both data rates (1 Mbps and 250 kbps) — confirm scope with your instructor; supporting both is recommended for full credit but a single-rate implementation is an acceptable reduced-scope option.

- ROM sizes and contents for: Walsh–Hadamard code tables, preamble/SFD, and the 4 chirp sequences (or the parameters needed to generate them).

- FSM states for the top-level controller that sequences preamble → SFD → PHR → payload → chirp modulation → done_Tx.

## 6. Verification Requirements

Verification is worth a significant part of the grade. You must show, with evidence, that your Verilog matches the fixed-point MATLAB model, and that the fixed-point model matches the floating-point model within the MSE bound.

- Unit testbenches: write a self-checking Verilog testbench for at least the Symbol Mapper, the DQPSK encoder, and the CSK Generator, each comparing against MATLAB-generated expected values (a stimulus/response file pair, or the MATLAB/HDL co-simulation approach described in Section 3.1.2 of the reference document).

- Integration testbench: a full-chain testbench that applies a payload, runs the simulation, and compares the final Tx_real/Tx_imag sample stream against the fixed-point MATLAB model output for the same payload.

- Accuracy report: compute the MSE between your fixed-point RTL output and the floating-point MATLAB reference, and show it is below the 0.005 threshold (or the threshold your instructor specifies).

- Behavioral (RTL) simulation: waveform screenshots/log showing correct bit-level operation of each block.

- Synthesis and implementation: synthesize the design for the target FPGA, report resource utilization (LUTs/slices, BRAM, DSP blocks) and the post-place-and-route timing summary (maximum clock frequency).

## 7. Tools

| Stage | Suggested Tool(s) |
| --- | --- |
| Floating/fixed-point modeling | MATLAB, Fixed-Point Toolbox (fi objects) |
| HDL design entry | Verilog, any text/HDL editor |
| Behavioral simulation | A Verilog simulator such as ModelSim/QuestaSim, Icarus Verilog, |
| MATLAB–HDL co-simulation | MATLAB/Simulink HDL Verifier (successor to the "EDA Simulator Link" described in the reference document) |
| Synthesis & Implementation | Xilinx Vivado / Altera Quartus |

## 8. Work Plan — Step-by-Step Guide

Follow the design flow in Figure 3-1 of the reference document. Suggested weekly milestones (adjust to your course calendar):

### Phase 1 — Study and Planning

- Read the reference document, Chapters 1 and 2, in full. Summarize, in your own words, the CSS PPDU structure and each processing block.

- Run the provided MATLAB floating-point model on a few example payloads and plot the resulting I/Q waveform and a chirp sequence, to build intuition before writing any Verilog.

- Run the provided fixed-point MATLAB model on the same payloads; compute the MSE against the floating-point output and confirm it is within the required bound.

- Draw your own top-level block diagram and module list, and a state diagram for the top-level controller FSM. Have this reviewed/approved before writing RTL.

### Phase 2 — RTL Design Entry

- Implement and unit-test each block in isolation, in the following recommended order: Symbol Mapper → Zero Padding/Demux → Interleaver → Preamble/SFD ROM → QPSK Mapper → DQPSK encoder → CSK Generator/chirp ROM → top-level controller/integration.

- For each block, write the HDL entity first (inputs/outputs/parameters), then a simple architecture, then a self-checking testbench with MATLAB-generated expected values, before moving to the next block.

- Keep a lab notebook / log of design decisions (word lengths chosen, ROM sizes, FSM states) — you will need this for your final report.

### Phase 3 — Integration and Behavioral Simulation

- Connect all blocks into the top-level transmitter module.

- Run the full-chain integration testbench against the fixed-point MATLAB model for at least 3 different payload lengths, including edge cases (payloadLength = 0 or 1, and the maximum 127 bytes).

- Debug any mismatches at the block boundary where they first appear, not by patching the top level.

### Phase 4 — Synthesis and Implementation

- Synthesize the design for the target FPGA board and resolve any synthesis warnings (inferred latches, unconnected ports, etc.).

- Run implementation (translate/map/place-and-route) and review the timing report; ensure your target clock frequency is met.

- Generate the bitstream and record resource utilization for your report.

### Phase 5 — Device Programming and On-Board Verification

- Program the FPGA and verify basic operation (done_Tx pulses, output samples change with payload).

### Phase 6 — Report and Demonstration

- Write the final report (see Section 9) and prepare a short live demonstration of the design running on the FPGA.

## 9. Deliverables

- All Verilog source files (one module per block, top-level integration, and package/constants files for ROM contents).

- All testbenches (unit and integration) and any MATLAB scripts used to generate expected values.

- Simulation logs/waveforms demonstrating correct behavioral simulation.

- Synthesis and implementation reports (utilization + timing).

- A written project report including: your block diagram, FSM diagram, word-length/ROM design decisions with the MSE justification, verification methodology and results, resource utilization, timing results, and a discussion of any known limitations.

- A short (5–10 minute) live or recorded demonstration of the design running on the FPGA.

## 10. Grading Breakdown

| Component | Weight |
| --- | --- |
| Block diagram / FSM design review (Phase 1) | 10% |
| RTL correctness — unit-level verification | 25% |
| Full-chain integration verification & MSE accuracy | 25% |
| Synthesis, implementation, resource/timing results | 15% |
| On-board demonstration | 15% |
| Final written report | 10% |

## 11. References

Primary reference (provided):

- "Zibee Reference Document" (attached) — CSS PHY floating-point model, floating-to-fixed-point conversion, and FPGA hardware implementation methodology.

From the reference document's own bibliography (consult the original standard and textbooks for full detail):

- IEEE Std 802.15.4-2003, Wireless MAC and PHY Specifications for LR-WPANs.

- IEEE Std 802.15.4a-2006/2007, amendment adding alternate PHYs (including CSS) — the normative source for the code tables, PPDU formats, and MSE acceptance criterion used in this project.

- J. Gutierrez, E. Callaway, R. Barret, "Low-Rate Wireless Personal Area Networks — Enabling Wireless Sensors with IEEE 802.15.4," IEEE Press, 2003.

- Nanotron, "nanoNET Chirp Based Wireless Networks White Paper," ver. 1.04, February 2007.

- John G. Proakis, "Digital Communications," 3rd ed., McGraw-Hill.

- Ata Elahi & Adam Gschwender, "ZigBee Wireless Sensor and Control Network," Prentice Hall, 2009.

- Bernard Sklar, "Digital Communications: Fundamentals and Applications," 2nd ed., Prentice Hall, 2001.

- Janick Bergeron, "Writing Testbenches: Functional Verification of HDL Models," Kluwer Academic, 2003.

- Pong P. Chu, "RTL Hardware Design Using VHDL: Coding for Efficiency, Portability, and Scalability," Wiley, 2006 (also relevant for general RTL design discipline even though your target is Verilog).

- Pong P. Chu, "FPGA Prototyping by Verilog Examples," Wiley — recommended companion text since it uses Verilog directly and the same class of Xilinx boards.

- Xilinx / AMD documentation for your specific target board (e.g., Vivado Design Suite User Guides, board-specific reference manuals).